Brazil_wind_data_10km - METADATA

Identification Information:

Originator: CEPEL (Electric Energy Research Center/Federal University of Rio de Janeiro) - Brazil Title: Brazil Wind Data (10km) Description: Annual average of the aeolic potential at 50m. Content: wind speed in m/s, power class (7 classes), power density in W/m2 and Weibull k value. Cells measure 10km Time period for which the data is relevant: 1 year Spatial Extend of Data: Brazil Bounding Coordinates: West Bounding Coordinate: 78°10' W East Bounding Coordinate: 28°30' W North Bounding Coordinate: 8°10' N South Bounding Coordinate: 36°10' S Access Constraints: No restrictions Use Constraints: Quoting the source is required: “CEPEL (Electric Energy Research Center/Federal University of Rio de Janeiro) - Brazil” Contact Information: Contact Organization: CEPEL - Electric Energy Research Center Contact Person: Antônio Leite de Sá Mailing Address: CEPEL - Centro de Pesquisas de Energia Elétrica Universidade Federal do Rio de Janeiro Avenida Um, s/n Ilha do Campus Universitário Rio de Janeiro - RJ - Brasil CEP 21941-590 Phone Number: +55 21 2598-6380 Email Address: alsa@cepel.br Website: http://www.cepel.br

Data Quality Information:

Lineage: The thematic map by code of colors permits quick viewing of all the Brazilian territory dataset. That map indicates, for the height of 50m, the annual average, in W/m2, of wind speed, power class, power density and Weibull k value. Attribute Accuracy: The information is organized into cells measuring 10 x 10km. The wind potential maps were calculated from simulations produced by the MesoMap(*) for 360 days, extracted of a period of 15 years of data. The days were choosen by means of random sampling at several heights, so that each month and season be considered in a representative way. (*) MesoMap is an integrated group of atmospheric simulation models, geographical and meteorological databases, nets of computers and storage systems. The MesoMap has been checked by high quality anemometric measurements in a large wind regimens range. Source Scale Denominator: 0

Spatial Data Organization Information:

Spatial Object Type: Vector - polygon

Spatial Reference Information:

Horizontal Coordinate Scheme: --- Horizontal Units: Decimal degrees Latitude Resolution: --- Longitude Resolution: --- UTM Zone Number: --- Map Projection Name: Polyconic Map Projection Parameters: Longitude of Central Meridian: 54° W Latitude of Projection Origin: 0° False Easting: 0 False Northing: 0 Other Coordinate System Definition: --- Cell Width: --- Cell Height: --- Geodetic Model: Horizontal Datum Name: SAD-69 Ellipsoid Name: Reference ellipsoid 1967 (International Astronomical Union)

Entity and Attribute Information:

Entity and attribute overview: Annual average of the aeolic potential at 50m. Cells measure 10km. Entity Label: Brazil_wind_data_10km Attribute Label: ID_CEL Attribute Definition: cell identification Attribute Label: LONGITUDE Attribute Definition: longitude of the cell center Attribute Label: LATITUDE Attribute Definition: latitude of the cell center Attribute Label: WEIBULL Attribute Definition: Weibull k value Attribute Label: SPEED Attribute Definition: wind speed in m/s Attribute Label: PW_DENSITY Attribute Definition: wind power density in W/m2 Attribute Label: PW_CLASS Attribute Definition: wind power class (7 classes)

Metadata Reference Information:

Metadata Date: July 20, 2004 Metadata Contact Organization: INPE - National Institute of Spatial Researches Metadata Contact Person: Enio B. Pereira Mailing Address: INPE-CPTEC Divisão de Clima e Meio Ambiente Rodovia Presidente Dutra, Km 40, SP-RJ Cachoeira Paulista - SP - Brasil CEP 12630-000 Phone: +55 12 3186-8581 E-mail: eniobp@cptec.inpe.br